Kugel Replacement

Scope

Remove existing Kugel and Base Stone; the GC will then install the new CBU provided Kugel and Base Stone (make all connections and perform startup of the water feature). GC will also be required to replace damaged storage tank, re-pipe plumbing (so that 100% water filtration occurs before fountain), replace equipment, and verify operation of system (needs to meet all Kusser recommendations). GC will be responsible for all lifting equipment to move the Kugel. This will be a complete turnkey install.

Protect hardscape

Remove existing Kugal and Base Stone

Disposal of old Kugel and Base Stone

Remove surrounding concrete (if needed)

Make corrections or repairs to plumbing

Compaction as needed (to prepare to receive new Base Stone)

Remove existing water storage tank

Make needed improvements (per engineered drawings) for the new storage tank

Install new storage tank

Rework plumbing (100% filtered water feeding Kugel)

Install low water alarm (audible)

Install bypass valves, check valves, and throttling valves

Install new equipment - pump, cartridge filter, and backwash system

Verify filters and pump sizing prior to install

Install new gauges

Make all electrical connections

Service all make-up water components

Install new Kugel and Base Stone

Make all needed connections at Kugel

Replace colored concrete (to match existing)

Check water chemistry

Adjust low water alarm

Perform start-up and make final adjustments

*CBU will purchase and have delivered to site the Kugel and Base Stone from Kusser