Activity Hazard Analysis (AHA)

| Activity/Work Task: | Steel Erection / Assembly at Mezzanine | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | OMFE | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |
| Contractor: | Steel Erectors Anonymous | Severity | Probability |  |  |  |  |  |
| Date Prepared: XXXXXXX |  |  | Frequent | Likely | Occasional |  | Seldom | Unlikely |
| Prepared by (Name/Title): | Bill D. Steele | Catastrophic | E | E | H |  | H | M |
|  |  | Critical | E | H | H |  | M | L |
| Reviewed by (Name/Title): | Hansel Felipe | Marginal | H | M | M |  | L | L |
|  |  | Negligible | M | L | L |  | L | L |
| Notes: (Field Notes, Review Comments) |  | Review each “Hazard” with identified safety “Controls” and determine RAC (See above) |  |  |  |  |  |  |
|  |  | “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent, Likely, Occasional, Seldom or Unlikely. |  |  |  | RAC Chart |  |  |
|  |  | “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic, Critical, Marginal, or Negligible |  |  |  | E = Extremely High |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |
|  |  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  | M = Moderate Risk |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |

| Job Steps | Hazards | Controls | RAC |
| --- | --- | --- | --- |
| Small Tool Use (wrenches, drills, hammers, etc.) | Cuts / Soft Tissue Injury | - Wear gloves 100% of the time | L |
|  | Damage to Eyes | - Wear ANSI Z-87 rated safety glasses | L |
| Material Handling (moving steel / bolts / small equipment) | Back Strain | - Lift anything over 50lbs with a partner | L |
|  | Slips, Trips, and Falls | - Plan path of travel and ensure it is clear before moving any material | L |
|  | Cuts / Soft Tissue Injury | - Use gloves | L |
|  | Dropping material below | - If hoisting material to elevated surface, ensure it is correctly secured and away from edges | M |
| Cutting / Grinding / Welding Steel | Fires / Burns | - Fill out Hot Work Permit prior to work beginning - Clear area of any flammables - Wear suitable clothing that covers skin, welding jacket / pants if applicable | M |
|  | Fume Inhalation | - Only perform work in well ventilated area | M |
|  | Damage to Face / Eyes | - Use a face shield for cutting or grinding in addition to safety glasses - Use welding hood for welding | M |
| Accessing Top of Mezzanine | Falls | - Use an 8’ A-frame ladder and be very careful when climbing onto the mezzanine from there | M |
| Working on Top of Mezzanine | Falls | - Use harness and lanyard to tie off to square column since fall risk is greater than 6’ | M |
|  | Dropping Tools off Edge down below | - Make sure tools are secured and/or away from the edge | M |

| Equipment to be Used | Training | Inspection Requirements |
| --- | --- | --- |

| Activities Requiring a Competent or Qualified Person – Attach Proof of Competency |  |  |  |
| --- | --- | --- | --- |
| Activity |  | Designated Competent or Qualified Person |  |
| Ladder Use |  | Bill D. Steele |  |
| Fall Protection Use |  | Bill D. Steele |  |
| Signatures / Verification of Review |  |  |  |
| Name (Print) | Signature |  | Date |

| AHA Modified and Reviewed |  |  |
| --- | --- | --- |
| Name (Print) | Signature | Date |